The Investigation of beam splitter for FIReTIP

2/12/2016

- Motivation:

The original beam splitter using to combine the Vertical polarized beam and horizonal beam will cause high energy loss, as shown in Fig.1. The original mesh grid structure of the Beam splitter has 70% transmission and 30% reflection at 119 μm and 45° incidence with the 500 lpi mesh, as shown in Fig.2. As a result, 70% power from f1 and 30% power from f2 will loss in this setup.

Wire grid Polarizer could achieve high reflection for E∥wires and high transmission for ⊥wires, which is the ideal beam splitter to combine two lasers with orthogonal polarization.

Figure 1. Schematic of the FIReTIP laser arrangement, laser tuning parameters, and mixer output. (Page 45, Robert’s thesis1)

Figure 2.Metallic mesh beam splitter. FIReTIP has two-line densities to choose from, 500 and 400 lpi. (Page 103, Robert’s thesis1)

- Grid structure requirement

To avoid high diffraction, the grid period should be less than the wavelength, which means Λ≪λ. For 500 lpi, the grid period Λ≈50.8 μm and λ=119 μm, maybe good for beam splitter, but it would be better if the structure has Λ<50 μm.

- Business Investigation

Two company sale the Wire grid Polarizer

- Microteach

G40-S, Unit price: $1500

Wire diameter / Wire spacing (μm): 10 / 40

Clear aperture / External diameter (mm): 40 / 50

The transmissions with different polarization are given in Fig.3 and Fig.4.

Figure 3. Transmission with E vertical to wire direction,  from datasheet online

Figure 4. Transmission with E parallel to wire direction

- REFLEX Analytical Corporation

Tungsten 10µm Diameter ,25µm Spacing, unit price $1015.(https://www.reflexusa.com/wigrpo.html)

Outside Diameter/Clear Aperture/Part Number: 50mm/25mm/57201

From Fig.5 Curvature A measured at 45o, when E parallel to the wire direction, the reflection at 119 um is about 95%.for E vertical to the wire direction, almost 100% transmission as seen from Fig.4.  G25 refers to 25 um spacing

Figure 5. data from online

Beam splitter

- Development of Laser Based Plasma Diagnostics for Fusion Research on NSTX-U, ROBERT ADAM BARCHFELD.