Stark laser testing plan

Equipment: power detector*2, NI-data acquisition, beam splitter*2, TIReTIP LabVIEW platform, Velemx controller.

- Power measurement

Previous testing:

FIReTIP: pressure 1-4 mTorr, Power 60-70 mW.

Stark laser: pressure 60-80 mTorr, Power 10 mW.

- Frequency shift measurement

Check the frequency shift under different cavity displacements.

| Cavity shift (um) | IF (MHz) |
| --- | --- |
| 0.16 |  |
| 0.32 |  |
| 0.48 |  |
| 0.64 |  |
| 0.8 |  |

Preparations:

- Check the LabVIEW code

- Check the NI DAQ

- Check the power meter

- Check the motor control

- Check the mixer

- Find the attenuator between the mixer and the spectrum analysis.

Frequency shift Assessment:

Consider the main wavelength λ0 corresponding to f0. when the cavity length of stark laser is adjusted to L0 to meet the resonant condition, we have

L0=Nλ02#1

Since the FIReTIP frequency bandwidth is about f0± 2MHz, for wavelength λ slightly away from λ0 , if the cavity length is a slightly shifted away from L0 but still have the same integer number N at resonant condition, we have

L=Nλ2#2

The cavity shift distance is

Δd=L-L0=N2λ-λ0#3

Finally, substituting eq. (1) into eq. (3):

Δd=-L01-λλ0=-L01-f0f=-L0Δff=-L0Δff0+Δf#4

where Δf=f-f0 , and by expanding Eq. (4) using a Taylor series, we obtain:

Δd=-L0Δff01+Δff0+12!Δff02+oΔff02#5

Since Δf<1 MHz and f0=2.5THz, Δff0<1e-6, so eq. (5) can be simplified to

Δd=-L0Δff0#6

So the relationship between the cavity shift and the frequency drift is

Δf=-Δd*f0L0#7

Increasing the cavity length would cause a downshift in the frequency.

For our system, the L0≈2m, f0≈2.5 THz,  the frequency drift with scanning step size is shown in figure. An increase of 0.1 µm in cavity length can lead to a frequency downshift of approximately 130 kHz. now the minimum cavity scanning step size is 0.16 um.

Moreover, the frequency cannot increase or decrease indefinitely with cavity tuning; the maximum frequency variation is constrained by the spectral bandwidth of methanol gas at 2.5 THz, which is governed by the quantum structure of the gas molecules. Outside of this bandwidth range, there will be no power, or the system may transition into another N±1 resonant condition.