# ECE Development for Next Step Fusion

Calvin Domier, 31 August 2026

Assumption: RF bandwidth of individual channels ≈ 400 MHz

This allows the use of Mini-Circuits LFCN-120+ low-pass filters in the standard UC Davis double downconversion approach. These filters have a 3-dB cutoff of 195 MHz and a 30-dB cutoff of 265 MHz.

To minimize LO interference, the ECE IF frequencies are chosen to maintain > 30 dB attenuation on any LO pickup signals. This is only possible with slight adjustments to a constant RF channel spacing. Interference signals for TCV exist at 9.6875 and 9.4375 GHz. Interference signals for DIII-D exist at 10.40 and 11.9375 GHz.

If the IF channel spacing is allowed to vary from 1.40 to 1.52 GHz, we get channels at 2.10 GHz, 3.50 GHz, 4.90 GHz, 6.30 GHz, 7.70 GHz, 9.16 GHz, 10.68 GHz, and 12.20 GHz.

With these channels and a 400 MHz RF bandwidth, the actual IF coverage spans 1.90 GHz to 12.40 GHz. Extending down to the 30-dB point, the IF coverage increases to 1.83 GHz to 12.47 GHz.

An option here is to allow for adjustability in the VCO settings for channels 6, 7, and 8, which could allow easy re-tuning for TCV with IF frequencies of 9.10 GHz, 10.50 and 11.90 GHz. This would permit a constant 1.40 GHz RF channel spacing and would be my preferred option.

VCO selection listed below, with all components in stock as of today.

Channel 1: 2.10 GHz, HMC384LP4E

Channel 2: 3.50 GHz, HMC390LP4E

Channel 3: 4.90 GHz, HMC429LP4E

Channel 4: 6.30 GHz, HMC358MS8GE

Channel 5: 7.70 GHz, HMC532LP4E

Channel 6: 9.10-9.16 GHz, HMC1163

Channel 7: 10.50-10.68 GHz, HMC1166

Channel 8: 11.90-12.20 GHz, HMC1166